Self-priming electric pump with bronze gear for transfer of various liquids, complete with in-line filter. Pump body made of brass with stainless steel shaft and a rubber lip seal. Ports: 3/8" BSP with brass connection for Ø 14 mm inner tubing. Main applications: transfer of diesel fuel, water, antifreeze etc.

| CODE | TYPE | VOLTS | FUSE | FLOW RATE | LIFT | WEIGHT kg | DIMENSIONS | Units in one pallet – Kg. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 06102 | UP3 | 12 | 10 A | 14 L/Min | 20m | 1.5 | 140X160 | 12 unds x 30 boxes=360unds – 600 kg. |
| 06103 | UP4 | 24 | 5 A | 14 L/Min | 20m | 1.5 | 140X160 | 12 unds x 30 boxes=360unds – 600 kg. |

Self-priming electric pump with bronze gear for transfer of various liquids complete with in-line filter. Pump body made of brass with stainless steel shaft and a rubber lip seal. Ports: 1/2" BSP with brass connection for Ø 16 mm inner tubing. Main applications: transfer of diesel fuel, water, antifreeze etc.

| CODE | TYPE | VOLTS | FUSE | FLOW RATE | LIFT | WEIGHT kg | DIMENSIONS | Units in one pallet – Kg. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 06112 | UP6 | 12 | 15 A | 26 L/Min | 20m | 3.1 | 183X174 | 6 unds x 30 boxes=180unds – 600 kg. |
| 06113 | UP6 | 24 | 7.5 A | 26 L/Min | 20m | 3.1 | 183X174 | 6 unds x 30 boxes=180unds – 600 kg. |

Self-priming electric pump with bronze gears for heavy duty operations whenever a prolonged service or a pressure of maximum 4 BAR is required, complete with in-line filter. Pump body made of brass with stainless steel shaft and a rubber lip seal. Ports: 3/8" BSP with brass connection for Ø 14 mm inner tubing. Main applications: transfer of diesel fuel, water, antifreeze, etc.

| CODE | TYPE | VOLTS | FUSE | FLOW RATE | LIFT | WEIGHT kg | DIMENSIONS | Units in one pallet – Kg. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 06142 | UP9 | 12 | 15 A | 12 L/Min | 40m | 2.7 | 166X170 | 6 unds x 30 boxes=180unds – 600 kg. |
| 06143 | UP9 | 24 | 7.5 A | 12 L/Min | 40m | 2.7 | 166X170 | 6 unds x 30 boxes=180unds – 600 kg. |